# RV-14 First Engine Run — Test Preparation

Aircraft: Vans RV-14 N8114X

Engine: Lycoming YIO-390-EXP119 / EXP289

Ignition: Dual P-Mags

Propeller: Hartzell Constant Speed

Avionics: Garmin G3X / VP-X Pro

Battery: EarthX ETX1200

## Test Objectives

- Safe Engine Start

- Oil Pressure Rise

- No Fuel, Oil or Exhaust Leaks

- Stable Engine Operation

- Verify Engine Instrumentation

- Verify Engine Control Operation

- Verify Ignition System

- Verify Prop Governor

- Verify Alternator Charging System

- Verify Normal Temperature Rise

- Normal Engine Shutdown

## Configuration:

- Aircraft Secured

- Cowl removed

- Observer with fire extinguisher

- Camera installed

- G3X Logging Enabled

## Possible Emergencies:

- Engine start malfunction

- Fire

- Emergency shutdown

### Engine Start Malfunction

- Starter | Release

- Mixture | ICO

- Fuel Pump | Off

- Ignitions | Off

- Fuel Selector | Off

- Battery | Off

### Fire On Engine Start

- Mixture | ICO

- Fuel Selector | Off

- Starter | Continue Cranking

- Evacuate

### Fire During Ground Run

- Mixture | ICO

- Fuel Pump | Off

- Fuel Selector | Off

- Ignition | Off

- Battery | Off

- Evacuate

### Abnormal Engine Operation / Emergency Shutdown

- Throttle | Idle

- Mixture | ICO

- Ignitions | Off

- Battery | Off

- Fuel Pump | Off

- Fuel Selector | Off

## First Start Procedure

### --- Pre-Oil Procedure ---

- Verify correct ignition lead configuration

- Add oil

- Remove bottom spark plug from each cylinder

- Disconnect all spark plug leads

- Fuel off

- Mixture Idle

- Throttle Full Open

- Ignition Off

- Crank (7 second increments) - 1 minute cool-down

- Crank until 20 PSI observed

- Reinstall Spark Plugs

- Reconnect all spark plug leads

- Perform engine start within 3 hours

### --- Before Start ---

- Preflight Inspection | Complete

- Passenger Briefing | Complete

- Camera | Recording

- Canopy | Taxi

- Fuel Selector | Fullest Tank

- Ignition | Off

- Electrical Switches | Off

- Alternate Air | Off

### ---Boot Sequence---

- Throttle | Idle

- Prop | Full Forward

- Mixture | ICO

- Standby Battery Switch | On

- Verify PFD Starts Normally | Check

- Main Battery Switch | On

- Alternator Switch | Off

- Avionics | On

- PFD | Verify no reboot

- PFD | Verify no failures nor annunciations

- PFD | Split Screen with Engine Data

- PFD | Verify >12V

- PFD | Verify Fuel Quantity

- PFD | Set Radio

- Time | Note

### --- Prime and Start Sequence ---

- Fuel Pump | On

- Throttle | Full Open

- Mixture | Full Rich

- Fuel Flow | Observed

- Throttle | Closed

- Mixture | Idle Cutoff

- Fuel Pump | Off

- Throttle | 1/4 Inch Open

- Prop Area | Clear

- Left & Right Ignition | On

- Starter | Engage

- Mixture | Advance

- Oil Pressure | Check 50-90 PSI in <30 sec

- Flaps | Up

- Mixture | Lean for peak EGT

- Idle at 1000 RPM for 3 minutes

- Observe Engine Gauges:

- Oil Temp - Max: 235 F - Desired Range: 165 - 200 F

- CHT: 150-250 F range

- EGT: Look for similar numbers on all cylinders 900-1300 F

- Voltage: Note range (12.5-13.2V expected)

### --- Runup (3 minutes after start) ---

- Alternator | On

- Fuel Pump | On

- Throttle | 1800 RPM

- Observe Engine Gauges:

- Oil Temp - Max: 235 F - Desired Range: 165 - 200 F

- CHT: 150-250 F range

- EGT: Look for similar numbers on all cylinders 900-1300 F

- Voltage: Note range (13.8-14.4V expected)

- Turn on electrical load (lights)

- Voltage: Remains in recharging range (13.8-14.4V)

### --- Ignition Test---

- Left Ignition | Test (No RPM Drop)

- Left Ignition | On

- Right Ignition | Test (No RPM Drop)

- Right Ignition | On

- Left Ignition | Off (150 RPM Drop)

- Right Ignition | Test (engine continues running)

- Right Ignition | On

- Left Ignition | On

- Right Ignition | Off (150 RPM Drop)

- Left Ignition | Test (engine continues running)

- Left Ignition | On

- Right Ignition | On

### ---Prop Check---

- Propeller | Cycle

- Verify Oil Pressure Drop

- Note, < 500 RPM Drop

### ---Engine Instruments---

- Engine Instruments

- Oil Temp - Max: 235 F - Desired Range: 165 - 200 F

- CHT: <380 (no more than 400)

- EGT: 1400 range

- Fuel Flow: 4-7 Gal/H

### --- Engine Shutdown ---

- Throttle | Idle

- Fuel Pump | Off

- Throttle | Idle

- Mixture | ICO

- Fuel Selector | Off

- Left & Right Ignition | Off

- Main Battery | Off

- Alternator Switch | Off

- Avionics Switch | Off

- Electrical Switches | Off

- Standby Battery Switch | Off