# RV-14 Taxi Test

Aircraft: Vans RV-14 N8114X

Engine: Lycoming YIO-390-EXP119 / EXP289

Ignition: Dual P-Mags

Propeller: Hartzell Constant Speed

Avionics: Garmin G3X / VP-X Pro

Battery: EarthX ETX1200

## Test Objectives

- Safe Engine Start

- Oil Pressure Rise

- Brake effectiveness

- Differential braking effectiveness

- Tail wheel steering

- Tail wheel unlocked turn

- Tracking true

- Magnetometer calibration (page 30-24)

- Successful shutdown

- Post-run inspection

## Configuration:

- Cowl installed

- Fire extinguisher on board

- Camera installed

- G3X Logging Enabled

## Pre-Conditions:

- ADAHRAS configured (page 30-10)

  - Note – accept the vibration test without engine running. Perform the vibration test later if issues arise. This will minimize engine run time during break-in period.

- GMC 507 Calibrated

## Possible Emergencies:

- Engine start malfunction

- Fire

- Emergency shutdown

### Fire On Engine Start

- Mixture | ICO

- Fuel Selector | Off

- Starter | Continue Cranking

- Evacuate

### Fire During Ground Run

- Mixture | ICO

- Fuel Pump | Off

- Fuel Selector | Off

- Ignition | Off

- Battery | Off

- Evacuate

### Abnormal Engine Operation / Emergency Shutdown

- Throttle | Idle

- Mixture | ICO

- Ignitions | Off

- Battery | Off

- Fuel Pump | Off

- Fuel Selector | Off

## Brake Failure

- Pump brakes to restore pressure

- Throttle | Idle

- Mixture | ICO

- S-turn as needed

## Magnetometer Calibration Failure

- Restart procedure

- If fails twice, abort test

# Test Sequence

Install cowling

Calibrate ADAHRS and GMC507

Pre-flight

Position aircraft on taxiway

Start engine

Immediately taxi towards Compass Rose

Check steering and brakes on apron

Perform magnetometer calibration

Taxi back to hangar

Shutdown

Inspection

Debrief

- All objectives were achieved

- Additional notes:

  - Com1 check - 5/5 with Ramona Ground

  - Oil pressure seemed to drop throughout the magnetometer calibration where throttle remained 1200-1300 RPM. 50 PSI at the end, whereas the pressure started at ~60 PSI. Throttling up on taxi back, pressure rose back to 60+. After initial start, pressure was 80-90 PSI. Need to check for any signs of leaking oil during post-flight inspection

  - Lower idle RPM - ~1000 RPM – the engine seemed to be missing intermittently

  - More “hot engine” smell in the cockpit. Possibly due to Cowling installed

  - CHTs seemed to remain below 350 F – need to verify

  - Brakes somewhat weak, as expected. Plenty of brake pressure to hold AC at 1300 RPM (max), and to slow and stop AC on taxiway

  - Possibly a backfire or two observed during magnetometer calibration – check video audio

  - ATT Warning on initial boot – possibly related to the ADHRS calibration?

  - Taxi felt normal, much like the RV-7 in terms of noise and feel.

  - No noticeable pull left or right, tracked straight

  - Rudder pedals are ok in terms of their position. Tiny bit closer would be good, although I’m able to fully engage the brakes in their current position.

  - By stretching my back I could see over the top of the cowl. May benefit from a slightly higher seat cushion.

  - More canopy space to the left of the cowling than I remember from the RV-7. Could see straight down the taxiway without touching the canopy

  - Ran from the left tank only, didn’t switch tanks

  - Magnetometer calibration was successful on first attempt.

  - Timeline – G3X data:

    - 9:10 – engine start recorded

    - 9:16:04 – logging stopped (accidental shutdown?)

    - 9:26:21 – logging restarted (after G3X reboot post calibration?)

    - 9:29:23 – Engine shutdown recorded

    - Total probable duration: 19 minutes, minus time shutdown on compass rose (1 minute?)

    - Total time so far = 19 + 9 = 28 minutes (0.4)

  - TODO: Test that the ignition “Test” function is actually working correctly – shut off main battery and alternator while engine is running, and verify that the engine keeps running at higher-than-idle RPM

  - TODO: Add a checklist for a hot engine start…

  - TODO: Clean the windshield to clear off oil and dust

  - TODO: Look into a 1-2 inch seat cushion to adjust view angle

  - The big “gotcha”

    - Attempting to boot into Calibration mode, I accidentally shut down the engine and needed to perform a hot start.

    - I believe the root cause was insufficient RPM for the Mags to power themselves, and they shut down when I shut off the Main battery + alternator.

    - Sequence:

      - Stopped on the compass rose

      - Throttle back to idle (or nearly)

      - Shut off the standby battery

      - Shut off the main battery. Surprisingly all avionics continued to function (good test of the alternator I guess)

      - Shut off the alternator, engine died

      - My initial reaction was to try to save it with power, but then remembered it’s better to let it shut down.

      - I proceeded to boot up the PFD into Calibration mode

      - I then performed an impromptu hot start: no fuel pump, throttle cracked, mixture at ICO

      - Cranked starter, and advanced mixture – engine caught and restarted normally.

      - Note – in Calibration mode, the MFD showed engine data. This was very useful as I performed the magnetometer calibration via the PFD